	[image: image2.jpg]FROMM | sistevs”




	Datasheet MH600
Plastic friction weld strapping head



All electric high tension plastic friction weld strapping head
The MH600 strapping head has been designed for automatic heavy duty industrial PET strapping installations. The head processes strapping from 13 – 19mm of various thicknesses and qualities. The high strap tension feature (up to 7000N) allows applications of PET strapping with break strength up to 12000N (2640 lbs).
[image: image3.png]615





Technical data for strapping head MH600
	Suitable strapping qualities:
	Polyester (PET) and Polypropylene (PP)

	Strapping dimensions:
widths


thicknesses
	12.0 – 19.0 mm  /  ½ - ¾”

0.60 – 1.35 mm  /  0.023 – 0.053”

	Average seal strength:
	up to 85% of strap breaking strength (dependent on quality)

	Sealing method:
	Friction weld

	Strap tension:
	7000N  /  1570 lbs adjustable

	Feed/retract speed:
	5.3 m/s  /  17.3 ft/s

	Tension speed:
	105 mm/s /  0.34 ft/s

	Pack width:


	min. 
W150 
x  H120 mm

max. 
W3000 
x  H3000 mm

	Power supply:


	400V – 3PE – 50Hz
230V – 3PE – 60Hz

	Consumption:
	1.1KW

	Control Voltage:
	24V DC

	Weight:
	104 kg  /  230 lbs


 Chart of types strapping head MH600

	Item No.
	Model
	Strap widths
	Strap thicknesses
	Max. Tension

	
	
	mm
	inch
	mm
	inch
	N
	lbs

	47.5601
	MH600/13-19/0.64-0.75/7.0/PET
	13-19
	½-¾
	0.64-0.75
	.025-.030
	7000
	1570

	47.5602
	MH600/13-19/0.80-1.35/7.0/PET
	13-19
	½-¾
	0.80-1.35
	.031-.053
	7000
	1570

	47.5603
	MH600/13-19/0.64-0.75/2.5/PET
	13-19
	½-¾
	0.64-0.75
	.025-.030
	2500
	560


Features

· Compact and modular design
· One piece of aluminium cast housing
· High strap tension up to 7000N (1570 lbs) adjustable
· Wide strap width range without changing parts
· Strap guide open feature
· Well proven friction weld sealing system
· High consistent seal efficiency because of environment temperature monitoring
· Dust cleaning system
· Three side suspension point for easy integration
Options

· De-coiler for Jumbo-coils
· Sword
· Telescopic insertion frame
· Movable machine
· Side sealing machine
· Head suspension with floating system
· Edge protection feed systems top and bottom
· Timper press system with various magazines
· Vertical and lateral press systems up to 80 KN
· Conveying systems
· Turntable for cross strapping
· Mechanical pack height sensor
· Re-inforced base plate
[image: image1.jpg]


[image: image2.jpg]


